3.5.14  Modem init.

This string is a Hayes modem command string that is automatically sent

to the modem when Novaterm first boots up.  It is also sent when you

press C= I in terminal mode.  A typical initialization string looks

like this:

ATE1M1Q0L2S11=50

This string would turn echo on, turn on the speaker, turn off quiet

mode, set the speaker volume to 2, and set the touch tone sound time

to 50 milliseconds.

3.5.15  Baud rate.

The baud rate is the speed at which data is sent and received over the

modem.  Novaterm can operate at 2400 baud under normal conditions and

as high as 38,400 baud using the SwiftLink(tm) cartridge.  You select

the baud rate from a menu of available baud rates which is limited by

the maximum baud rate of your modem.

Without a SwiftLink cartridge connected, Novaterm uses the RS-232 NMI

routines from the February 1989 issue of Transactor magazine.  These

routines provide clean, reliable communication at high speeds even

when the computer is sending and receiving data at the same time.

These routines are necessary because the RS-232 routines present in

the stock Commodore 64 do not work properly beyond 300 baud.  The

Transactor routines use carefully selected values for start-bit and

full-bit transmission times to overcome the errors.  These "times" are

actually values used by an internal counter to determine when the next

bit of data should be sent or received.  Each byte of data has eight

bits which must be sent plus a starting bit which precedes it. The

start bit takes slightly longer to send than the bits of real data

(called "full bits") and thus the start-bit time is longer than the

full-bit time.

3.5.16  Dial method.  See section 2.3.7.

3.5.17  Redial time.

During each attempt at autodialing, Novaterm waits a certain number of

seconds before giving up without a connection.  That amount of time is

set here.

3.5.18  Line delay.

This option specifies the number of seconds to wait between lines when

transmitting a textfile with the "Textfile to modem" or "Buffer file

to modem" options.

3.5.19  XOFF/XON characters.

These are single ASCII characters used to pause and resume

transmission.  Usually, XOFF is the ctrl-S key (ASCII 19) and XON is

ctrl-Q (ASCII 17) (these are Novaterm's defaults), but they may vary

among bulletin boards (for example, some BBS's use P to pause and any

other key to continue).  The ASCII values of the characters used by

Novaterm are displayed.  When you choose to modify either of these,

type the actual character to be used, not the ASCII value.

3.5.20  Flow control.

Since both 40 and 80 column modes may have trouble keeping up with baud

rates of 2400 baud and above, flow control has been implemented to

prevent character loss.  When the terminal is falling too far behind in

displaying incoming data, Novaterm tells the remote computer to pause

until Novaterm catches up.  When the terminal is nearly caught up, it

tells the remote computer to resume transmission.  This prevents

characters from being lost when Novaterm gets too far behind the

incoming data.  The flow contro l is done automatically, but it may be

turned off by toggling this option.

Novaterm supports two types of flow control.  One type is XOFF/XON,

where Novaterm sends an XOFF character (usually ctrl-S) to pause and an

XON character (usually ctrl-Q) to resume.  Because this type of control

involves sending characters to the remote computer, it is also called

software flow control.

Another type is RTS/CTS handshaking.  In this method, Novaterm actually

tells the modem to stop delivering data to your computer by clearing the

RTS line (one of the RS232 pins).  In this state, the modem stores

incoming data in a buffer until Novaterm re-asserts the RTS line.  In

addition, the modem can tell Novaterm to stop giving it data by clearing

the CTS line.  Because this type of control involves manipulating RS232

pins, it is also called hardware flow control.  It is much more precise

and reliable than XOFF/XON control, since modems respond immediately to

changes in RS232 lines.  However, to use this type of control, you MUST

have at least a 9600 bps modem with an RS232 connection.  Slower

Commodore modems do not respond to the RTS and CTS lines.

If you are using a high speed modem (4800, 9600 or 14.4Kbps) you MUST

use RTS/CTS flow control.  If you specifcally have a 14.4Kbps modem, set

Novaterm's baud rate to 19200, and RTS/CTS handshaking will control the

flow of data between your computer and modem.

A tolerance value for the flow control may be adjusted from the same

menu.  A higher number means Novaterm waits until it is further behind

before sending a pause character.  The default is 150 in an acceptable

range of 11 to 255. Generally, the higher the baud rate, the smaller

this number should be.  This is because there is a delay between the

time Novaterm realizes the buffer has reached its tolerance and the time

it pauses the remote, during which additional data may be received and

stored in the buffe r.

If you are using RTS/CTS flow control, you should get instant reponse,

and this tolerance value should be high.  With the inferior XOFF/XON

control, however, you may need to adjust the tolerance.

If you are operating at less than 2400 bps, the flow control tolerance

shouldn't concern you at all.  The problem occurs at 2400 bps or higher

in 80 column mode.  (You should already be using "Triple scroll" in this

case; see 3.1.2, Commodore key commands, C= W).  If you are still losing

a lot of characters even with flow control turned on, you should

decrease the tolerance until the character loss is a minimum.  This

number may be around 100 to 200 if you are directly connected to a

mainframe system.  It m ay need to be lower if you are on a BBS.  It all

depends on how long the remote system takes to respond to a pause signal

and on what speed you're using.

Novaterm automatically sets the flow control tolerance to a suggested

value when you or the program sets the baud rate.  These default values

are: 150 for 2400 bps, 100 for 4800 bps, 75 for 9600bps, 50 for 19200

bps, and 30 for 38400 bps.

On some mainframes, if you dial into the system with your modem at 2400

bps or less, you may find that pause signals won't have any effect at

all, and flow control does not work.  This is because the mainframe

system is sending data as fast as it can into its modem's buffer, and

its modem is emptying its buffer at the slower baud rate. Therefore, the

mainframe is already waiting at the prompt while its modem sends data.

To correct this situation, you must find some way to slow down the

mainframe to the spe ed of your connection.

Your mileage will vary with XOFF/XON flow control.  With some

connections it works flawlessly, while on others you may still lose

characters.  You should always use "Triple scroll" (see 3.1.2, Commodore

key commands, C= W) at 2400 bps or higher to prevent losing lots of

data.

3.5.21  Flow control tolerance.  See section 3.5.20 above.

3.5.22  Protocol block size.

The Zmodem, Punter, and Kermit protocols allow a variable block size

(see section 3.5.2, Protocols).  This option allows the block size to

be set within an acceptable range.  The value here may not be changed

when one of the other protocols (Xmodem, Xmodem-1k, Ymodem batch) is

in memory.

3.5.23  Comm parameters.

Some bulletin boards or mainframes may use only seven of the eight

bits of a byte for communication.  To accomodate for this, you may use

either 8 bits with no parity (8N1), 7 bits with even parity (7E1), or

7 bits with odd parity (7O1).  Most systems use 8 bits with no parity.

If this option is set to "8N1" when the BBS you are calling requires

"7E1", about half of the letters received will show up correctly and

half will appear as garbage.

3.5.24  Bell.

This simply turns the bell on or off.

3.5.25  Save buffer when full.

More than 8192 bytes of text may be captured and saved to disk without

running out of memory, even though the buffer is restricted to that

size.  When the buffer becomes full during a text capture, Novaterm

immediately sends a character that tells the host computer to pause

(called the XOFF character), just as if you had pressed the pause key.

Once the host computer stops sending data, Novaterm saves the buffer

capture file to disk, appending it to the disk file if it already

exists.  The capture file in the buffer is erased and re-opened,

making all the memory available again.  Then, Novaterm sends a

character to tell the host to resume transmission (the XON character),

also as if you had "hit a key to continue" yourself.  The buffer

capture continues after this, and the process repeats once the buffer

fills up again.  When the text you want to capture has all been

received, you must manually save the text remaining in memory.

Novaterm allows you to append the text to the existing disk file. This

is all done automatically, but it may be turned off by toggling this

option.

If a disk error occurs while trying to save the capture file, Novaterm

does not send the XON character to resume transmission.  This leaves

the remote system paused so that you may correct the problem, save the

capture file manually, and resume the text capture where it left off.

3.5.26  Transfer translation.

If you are downloading a sequential file (textfile) from a bulletin

board, it may be in standard ASCII.  In order to change the file to

PETSCII while it is downloaded, you may turn this option on. This

causes the incoming file to be translated through the ASCII receive

translation table in memory before it is saved to disk.  Similarly, if

this is toggled on when you upload a file, the file is translated

through the transmit table before it is sent to the receiving

computer.

3.5.27  Chop X/Ymodem padding.

When you download a file using Xmodem, Xmodem-1k, or Ymodem batch, the

last block is padded with control-Z characters to increase its size to

the required 128 or 1024 bytes.  This leaves extra bytes at the end of

the file when it is saved.  If you have this option turned on, these

padding bytes at the end of the file are stripped, bringing the file

back to its original length.  This could conceivably corrupt a binary

file which has control-Z characters at its end, but such files are

uncommon.

3.5.28  Zmodem auto-download.

The Zmodem protocol has an auto-download feature.  Novaterm can detect

when a remote computer is trying to send files using Zmodem.  It

automatically starts a Zmodem download when it detects this signal,

provided you have already selected the protocol and brought it into

the computer's memory.  This option enables or disables the

auto-download feature.

3.5.29  SwiftLink.

The SwiftLink cartridge may be mapped to three different memory

locations.  This may be necessary if you have some other device hooked

up to the cartridge port that uses SwiftLink's default memory

location.  Consult the SwiftLink documentation for more information

about changing the mapping within the cartridge itself.

Selecting this option, toggles through three areas of memory.  Select

the area of memory that your cartridge is mapped to. The default is

$DE00, which should be used if you have no other special devices

hooked up.  You may map the SwiftLink to $DF00 and $D700 as well.

Mapping to $D700 is only possible with a Commodore 128 in 64 mode.